To change/verify Ethernet option on S8700 servers

Dial up and login to server (to lynix shell)

server@definity1>  sudo setnic –q eth0    (to see current option and what it is running)

(i.e.  dadmin@definity2> sudo setnic -q eth0

ie100 Driver: eth0 setting=Auto-negotiate, currently=100Mbps/Half Duplex ) 

Usage:

(setnic [-d] [-p] -m <10H|10F|100H|100F|AUTO> <interface>

                OR

        setnic [-d] [-p] -q <interface>   )                                     

server@definity1 > sudo setnic –m 100f eth0 (to set eth0 to 100 fixed)

WARNING!!  When the Ethernet port is changed, the server will do a switch to try and find the better service. It may do this a couple of times before settling down to the new optioned server (could take ~5 minutes) After settling down to the changed server then repeat the procedure on what is now the off-line server.

  The switching of servers CAN CAUSE the modem to go to sleep. Then the server cannot be accessed remotely. Tech needs to be on-site to either reset the modem or access the server and make the change on the off line server.

Steve Moore

X8508

