[image: image75.png]AVAYA




Patching Instructions for AvayaTM S8700 & S8300 Media Servers
[image: image74.png]AVAYA




Patching Instructions for AvayaTM S8700 Media Servers

CONTENTS

2Patching Instructions for Upgrades to Avaya S8700 Media Servers

Patching Instructions for New Installations of Avaya S8700 Media Servers
13
Patching Instructions for Upgrades to Avaya S8300 Media Servers
21
Patching Instructions for New Installations of Avaya S8300 Media Servers
25


Patching Instructions for Upgrades to Avaya S8700 Media Servers

An Upgrade means existing (previously installed) S8700 sites ONLY

Step
Instructions
Example Screens

1
Administrators should be informed of a translation freeze during the duration of applying the patch. 

NOTE:  To verify which software version you are running, select “View Software Version” from the media server web pages.  This can be found under the Server Configuration and Upgrades heading.  The media server web pages can be accessed using a web browser (see step 3).

NOTE: DO NOT REMOVE PREVIOUS PATCHES WHEN APPLYING THE MOST CURRENT PATCH !




2
From a SAT/ASA session, execute a “save translation” command; this will force a filesync so the standby media server has the latest translation file.  
[image: image1.png]T Ciese New Tenplte

Fi Use Tenplte

== | Generate Call Accour
Report
ExportData
Iport Data
4 Find and Replace

G Stat Ematon

wechon

s






3
Start a web browser and specify the media server modem PPP IP address as the URL if logging in over a modem interface. Login using the appropriate login.

OR 

If there is LAN connectivity to the media server, start a web browser and specify the DNS name or LAN IP address of the standby server as the URL.  Login using the appropriate login. 

OR

If you are on-site and do not have LAN connectivity to the media server, connect a laptop to the standby server with a crossover cable, start a web browser on the laptop and specify 192.11.13.6 as the URL and login using the appropriate login.


[image: image2.png]WARNING: This s
access is a violati
reasons. By proce

This site ermploys ¢
certificate” provide
certificate, you will
offered this certifics
Screen" for instruct

&

AVAYA

communication

Information you exchange with i site cannol be viewed o1
changed by athers. However,there is » problem with the sie's
secuiy cetiicate.

The secuty cartcate was issued by a company you have.
ot chosen ta st View the cerlficale to determine whether
youwant to st the cartiying autharty.

@ The secuity certcate date s valid

The name on the secuiy certicate is invald o doss ot
malch the name of the ste

Do you want o proceed?

e ViewCoticae

oses. Unauthorized
strafive and security

have a "security
loes not have this

s certficate. If you are
b. Click "About This





[image: image3.png]AVAYA

communication

WARNING: This system is restricted to authorized users for business purposes. Unauthorized
access is a violation of the law. This system may be monitored for administrative and security
reasons. By proceeding, you consen {o this monitoring.

Username: init

Password: f==<f






4
Perform this step only if this is an UPGRADE to a S8700 External Communications Controller (ECC) configuration (if you have G700’s with Local Spare Processors (LSP)s as part of this configuration), otherwise go to step 5:

If the configuration is an ECC with LSPs, stop the MV software on all LSPs before continuing.  This can be done by telneting to each LSP device and executing a “stop –afcn” from the command line.

NOTE:  If you need the IP address of the media gateways, execute the “list media-gateway” command from the SAT/ASA session. 

NOTE: An enhancement will be made to allow the user from the web to busy-out the LSPs instead of having to manually shut them down, but for now this is the workaround.
[image: image4.png]> Command Promp!
raftBchiccl> stop -—afen

=10l x]|






5
Perform a Backup of all data sets using “Backup Now” under the Data Backup/Restore heading on the media server web pages, which can be accessed from a web browser (see step 3). 

You can backup to an FTP server that is on the customer network or to a flash card.

NOTE: you cannot backup translations via e-mail due to customer restrictions on their e-mail servers. 
[image: image5.png]AVAYA

This Server: [1] saturn
Duplicate Server: [2] saturn2

Help
Log off

Telephony Administration
ort ASA
~Dowrload ASA
Data BackupRestore:
—Backup Now
~Schedle Backup
—View Backup Log
~ViewRestore Data
Alarms and Nofification
~View Current Alarms.
Server
“View Summary Status
~View Pracess Status
frchange Servers
—Busy Out Server
~Release Server
—-Shutdown This Server
—-Set Server TinefTimezone
Security
~EnableiDisatle Madem
ertStop FTP Server
—View Lizense Status
—nstal Licznse
—instal Avaya Authertication
—-Set LAN Securty
~Enatle Tripwire
ipwire Commands
—nstal vaya Root Certfcate
sump
Contigure Trap Destinations:
ertStop Master Agent
iagnostics
View Restart Causes

Backup Now

Data Sets:
¥ Avaya Call Processing (ACP) Translations
& Save ACP translations prior to backup
© Do NOT save ACP translations prior to backup
¥ Server and System Files
¥ Security Files

Backup Method:

IFTP User Name: [swadrin Password: ==
Host Name: [dura Directory: [e/swadmin/satum/trans
© Email  User Name: at Domain Name:
Mail Server:

¢ LocalPC Card  Retain data sets at destination

Encryption:
I Encrypt backup using pass phrase

Start Backup About This Screen






6
From the laptop, FTP the patch to the standby media server via manual FTP or using the media server web pages “Upload Files to Server (via browser)” command under the Miscellaneous heading.  If you are downloading from an external Avaya web site you can use the media server web pages “Download Files to Server (from web)” command under the Miscellaneous heading to complete this step. 

MAKE SURE TO USE BINARY TRANSFER IF MANUALLY FTP’ING THE FILE!
[image: image6.png]telease Server

fuddown This Server i
et Server Time/Timezone SeloadiRlIeitolsenien

Socurity
rableDisatle Modem This page allows data files to be loaded onto the Server from the machine that your browser is
StertiStop FTP Server running on. In one or more of the fields below, enter the file name(s) to be sent to the server

[CAOT.0.080.4-4227 tar.g _ Brawse.

Browse.
File(s) to upload:

StartiStop Master Agert Browse.

Diagnostics Browse.

Vigw Restert Causes
LOAD FILE About This Screen

Miscellaneous
nableiDisable Ethernet Swich Parts
Jload Files to Server (via browser)
owrioed Fes to Server (romi)
Server Configuration and Upgr

lew Software Versian
ontigure Server
st New Software Release

1PS Firmware Upgrades
few Seril Number()

few PS Version

owrioed PSIFirmware

few Dowrioad Status

clivate PSI Lngrace

lew Firmware Activation Stetus





[image: image7.png]Release Server
Shutdown This Server
Set Server Time/Timezane.
Security

-Enable/Disable Modem The following files have been successfully uploaded to the server.
Startsstop FTP Server

Upload Files to Server

File Size
01.0.0604-4227 tar gz | 2873403

About This Screen

Miscellaneous
- EnableiDisable Ethernet Switch Parts

Uploed Fils to Server (ia browser)

Downiaed Files ta Server (from wek)

Server Configuration and Upgrades.
“View Software Version

Contigure Server

install New Software Release.

Make Server Upgrace Permanert

igw Pertton Status






7
Choose  “Busy Out Server” under the Server heading on the standby media server web pages, and choose “Busy Out” on the bottom of the screen. 

NOTE: This might cause a short delay in command line response while the media server is being busied out.


[image: image8.png]AVAYA

This Server: [2] saturn2
Duplicate Server: [1] saturn1

Help
Log off

Telephony Administration

Alarms and Nofification
~View Current Alarms.
Server

Security
nable/Dissble Madem
Startsstop FTP Server

ipwire Commands
stall vaya Root Certficate
sump
~Configure Trap Destinations.
Startistop Master Agert
Diagnostics
View Restart Causes

Busy Out Server

SERVER STATUS

Duplicated?
Standby Busied?
Standby Refreshed?

Standby Shadowing
Duplication Link
Elapsed Time since Init/Interchange

saturn?

Mode: Standby
Major hlarms: yes
Minor hlarms: yes

Control Network: 4 / 4

Server Hardvare: okay

Processes: okay

yes
aff
up
04 00:02:35

lode

Major Alarms

Minor Alarns
Control Network
Server Hardvare

Processes

saturni

Aotive
yes
yes
i/
clkay
clkay

Clicking on busyout will busy oLt this server so long s itis not in the ACTIVE state.

Busy Out About This Screen




[image: image9.png]AVAYA

This Server: [1] saturn
[buplicate Server: [2] saturn2

Help
Log off

Telephony Administration
Start ASA
Downiaad ASA
Data BackupRestore:
Backup Now
Schechle Backup
View Backup Log
iewRestore Data
alarms and Notification
~View Current Alarms.
server
Vigw Summery Status
igw Process Stalus.
iterchange Servers
Busy Out Server -
Release Server
Shutdown This Server
Set Server Time/Timezane.
security
EnableDisable Modem
Startsstop FTP Server

Busy Out Server Results

This server is nov Out of Service

About This Screen






8
From the web pages on the standby media server, verify the system is busied out by choosing “View Summary Status” under the Server heading.  This should report the standby media server is busied out.  

If you are on-site you will also notice the server state LED will have changed from flashing green to flashing red to indicate a BUSIED state.
[image: image10.png]View Summary Status

This Server: [2] saturn2 SERVER STATUS

Duplicate Server: [1] saturn Duplicated? ves
Standby Busied? yes
telp Standby Refreshed? no
Log orr Standby Shadowing: att
Duplication Link: up
Telephony Administration Elapsed Tine since Init/Interchange: 2d 01:06:22  (curbs in)
Start ASA
Dowrioad ASA saturn2 saturnl
Dota Beckupmestore Node: BUSY OUT Node: Aotive
i Najor Alans: no Major Alams: yes
Minor Alams: yes Minor Alars: no
View Backup Log Control Network: 0 / 0 Control Network: 4 / 4
ViewiRestore Data Server Hardware: okay Server Hardware: okay
Alarms and Notification Processes: okay Processes: okay
“View Curert Alrms
Server
View Sunmary Status © Display repeatedly every [05 7] seconds

Midioiehiing View| | AboutThis Screen
Busy Ot Server
Rekeaso Server
Shtcon T Server
St Server TimefTimezone
Somurity

Enabepisabe Moder
Startsion 7 Server
View Lisense Setus
st Lcense
el Avaya Authertication
St Lan Securty
Cincta waya Roct Caticate
st
Confire Trap estinaons
StatSion actr Agert
Disgnostics

Viow Restart Causes
View SystemLoge
Viow Tamperaturehiotage






9
Start a telnet session to the standby media server.  

From the telnet session, change directories to /var/home/ftp by executing the following at the command prompt:

> cd /var/home/ftp

To verify the file was uploaded to the media server, execute the following from the telnet session:

> ls -ltr

This will list the files in this directory, the last file shown should be the file you just ftp’ed which will be in the format of <patch>.tar.gz


[image: image11.png][commandpromet e [= ]

iorosoft Windows 2000 [Uersion 5.00.21951

KC> Copyright 1985-2088 Microsoft Corp.
s\Otelnet 192.11.13.6





[image: image12.png][ Command Prompt - telnet saturn2 =10l x]|

raft@saturn2> cd /var/home/FED





[image: image13.png]/INNT\System32\telnet.exe =10l x]|

susers 2873403 May 1 15:19 B1.8.868.4-4227.tar.gz






10
In order to install the patch execute the following from the telnet session:

>  patch_show

This will show any previous patches installed on the system. 

NOTE: DO NOT REMOVE PREVIOUS PATCHES WHEN APPLYING THE MOST CURRENT PATCH !

Now to install:

>sudo patch_install <patch>.tar.gz

To verify the patch was installed:

>  patch_show


[image: image14.png][ Command Prompt - telnet saturn2

-1o) x|
raft@saturn2> patch_shou

Patches are shoun in the order applied

oad-Patch Id Status

raft@saturn> _





[image: image15.png][ Command Prompt - telnet saturn2 =10l x]|

raft@saturn2> sudo patch_install O1.8.060.4-4227.tar.gz





[image: image16.png][ Command Prompt - telnet saturn2

raft@saturn2> patch_show

=lolx|

Patches are shoun in the order applied
oad-Patch Id Status

1.9.960.4-4227 not applied one-way talk patch on ISDN trunk
Gith EC5@8 int

raft@saturn2>






11
In order to apply the patch, execute the following from the telnet session:

>  sudo patch_apply <patch>

NOTICE: you do not use the .tar.gz extension on the back of the file name

NOTE:  Ignore the indication that “/opt/ecs/sbin/drestart 1 4 command failed”

NOTE:  when you apply the patch, the standby media server only – will go through a software reset system 4.  This reset will NOT impact service because it is on the standby side.

You will experience a delay in response to typing on command line until the restart/reset has completed.
[image: image17.png]=lo/x|

raft@saturn2> patch_show

Patches are shoun in the order applied

Status Patch-description

1.
Gith EC5@8 int

not applied one-way talk patch on ISDN trunk

raft@saturn2> sudo patch_apply B1.8.868.4-4227
Successfully applied the patch
Executing post-patch commands
-3 Jopt/ecs/shinsdrestart 1 4 command passed







12
Once system response has returned,

from the telnet session, verify the patch was applied.    Execute the following:

>  patch_show

This should show the status of the patch as “applied”.
[image: image18.png]=lo/x|

Patches are shoun in the order applied
oad-Patch Id

1.9.960.4-4227
Gith EC5@8 int

one-way talk patch on ISDN trunk

raft@saturn> _






13
From the web pages on the standby media server, choose “Release Server” under the Server heading and release the standby back into service.


[image: image19.png]AVAYA

This Server: [2] saturn2
Duplicate Server: [1] saturn1

Help
Log off

Telephony Administration

Alarms and Nofification
~View Current Alarms.
Server

Security
nable/Dissble Madem
Startsstop FTP Server

Startistop Master Agert

Control Network: 0 / 0
Server Hardvare

Release Server

SERVER STATUS

Duplicated?
Standby Busied?

Standby Refreshed?

Standby Shadowing

Duplication Link

Elapsed Time since Init/Interchange

saturn?

Mode: BUSY OUT
Major Alarms: no
Minor hlarms: yes

clkay

Processes: okay

Clicking on release wil release this server from busyout only if it s currently i the busyout state or

if & busyout is pending (ot reacy).

Release About This Screen

yes
yes
aff
up
2d 01:07:19

lode

Major Alarms

Minor Alarns
Control Network
Server Hardvare

Processes

(curt
saturni

Aotive
yes
g
clkay
clkay

bs in)

4





14
From the web pages on the standby media server, choose “Interchange Servers” under the Server heading and select the “force” option at the bottom of the screen.  Then choose “Interchange”.  This will force the standby server to go active, and the active to become the new standby, by causing a reset system 4 on the system.  

This will cause a SHORT service outage while MultiVantageTM software is coming up.  You will not be able to get command line response nor web response while the software is initializing.


[image: image20.png]This Server: [2] saturn2
Duplicate Server: [1] saturn1

Help
Log off

Telephony Administration
Start ASA
Downiaad ASA
Data BackupRestore:
Backup Now
Schechle Backup
View Backup Log
iewRestore Data
Alarms and Nofification
~View Current Alarms.
Server
Vigw Summery Status
igw Process Stalus.
iterchange Servers
Busy Out Server
Release Server
Shutdown This Server
Set Server Time/Timezane.
Security
EnableDisable Modem
Startsstop FTP Server
View License Status
intall License
install Avaya Authertication
Set LAN Security
—nstal Avaya Root Certfcate
sump
Contigure Trap Destinations:
Startistop Master Agert
Diagnostics
Vigw Restert Causes
View System Logs
View Temperaturefvotage

Interchange Servers

SERVER STATUS

Duplicated?
Standby Busied?
Standby Refreshed?

Standby Shadowing
Duplication Link
Elapsed Time since Init/Interchange

saturn?

Mode: Standby
Major Alarms: no
Minor hlarns: yes

Control Network: 4 / 4

Server Hardvare: okay

Processes: okay

W Force interchange regardless of server status

About This Screen

yes
yes
up
2d 01:09:08

lode

Major Alarms

Minor Alarns
Control Network
Server Hardvare

Processes

saturni

Aotive
yes
g
clkay
clkay

4




[image: image21.png]AVAYA

This Server: [1] saturn
[buplicate Server: [2] saturn2

Help
Log off

Telephony Administration
Start ASA
Downiaad ASA
Data BackupRestore:
Backup Now
Schechle Backup
View Backup Log
iewRestore Data
alarms and Notification
~View Current Alarms.
server
Vigw Summery Status
igw Process Stalus.
iterchange Servers
Busy Out Server -
Release Server
Shutdown This Server
Set Server Time/Timezane.
security
EnableDisable Modem
Startsstop FTP Server

Interchange Servers Results

Interchange Confirmed

About This Screen





15
From the web pages select “View Summary Status” under the Server heading.  This should show the media server (previously the standby) as active now.


[image: image22.png]This Server: [2] saturn2
Duplicate Server: [1] saturn1

Help
Log off

Telephony Administration
Start ASA

Downiaad ASA

Data BackupRestore:
Backup Now

Schechle Backup

View Backup Log

iewRestore Data

Alarms and Nofification

~View Current Alarms.
Server

igw Summery St
igw Process St
irterchange Server!

Busy Out Server
Release Server
Shutdown This Server
Set Server Time/Timezane.
Security

EnableDisable Modem
Startsstop FTP Server
View License Status
intall License
install Avaya Authertication
Set LAN Security
—nstal Avaya Root Certfcate
sump
Contigure Trap Destinations:
Startistop Master Agert
Diagnostics

Vigw Restert Causes
View System Logs
View Temperaturefvotage

View Summary Status

SERVER STATUS

Duplicated? yes
Standby Busied? no
Standby Refreshed? yes
Standby Shadowing: on
Duplication Link: up
Elapsed Time since Init/Interchange: 0d 21:24:00

saturn? saturni
Mode: Active Mode: Standby
Major hlarns: yes Major Alarms: no
Minor hlarms: yes Minor Alarns: yes
Control Network: 4 / 4 Control Network: 4 / 4
Server Hardvare: okay Server Hardvare: okay
Processes: okay Processes: okay

© Display repeatedly every [05 7] seconds

View About This Screen





16
From the web browser, choose “View Software Version” from the Server Configuration and Upgrades heading, and verify the patch is shown applied to the system.  

This information is shown directly under the Release String in the middle of the page. 


[image: image23.png]Release Server
Shutdown This Server
Set Server Time/Timezane.
Security

EnableDisable Modem
Startsstop FTP Server

Miscellaneous
- EnableiDisable Ethernet Switch Parts

Uploed Fils to Server (ia browser)

Downiaed Files ta Server (from wek)

Server Configuration and Upgrades.
“View Software Version

Contigure Server

install New Software Release.

Make Server Upgrace Permanert

igw Pertton Status

View Software Version

Operating systen
Built

Contains
Reports as

Release String
01.0.060.4-4227

Translation Saved

License Installed

About This Screen

Linuz 2.2.17-14.1s15 1686 unknovn

Mar 8 16721 2002
01.0.060.4
RO11:.01.0.060.4
S8700-011-01.0.060.4
applied

May 113:13
Apr 17 16:19

one-way talk patch on ISDN trunks with E






17
If connected via laptop interface:

Now disconnect the laptop crossover cable from the current server (now active).  You are now not connected to any media server.  From the MS-DOS command window execute:

>   arp –d 192.11.13.6

This command flushes the route table in the laptop before connecting to the new standby server.

Connect the crossover cable to the other media server (the new standby) and telnet to this server by executing the following from the MS-DOS command window:

> telnet 192.11.13.6

NOTE:  Be patient if this connection is slow; give it a few minutes.

OR 

If connecting via customer LAN:  

Telnet or connect to the new standby server over the customer’s LAN IP address 

OR

If connecting via the modem:

From the current session execute the following from the server prompt:

> telnet <server_name>-dup


[image: image24.png]KC> Copyright 1985-2088 Microsoft Corp.

-1o) x|
\varp —d 192.11.13.6





[image: image25.png][commandpromet e [= ]

iorosoft Windows 2000 [Uersion 5.00.21951

KC> Copyright 1985-2088 Microsoft Corp.
s\Otelnet 192.11.13.6






18
Repeat steps 5-13 for the new standby media server. After the last release in step 13 be sure to check that the servers are refreshed and the standby is no longer busied.  


[image: image26.png]This Server: [2] saturn2
Duplicate Server: [1] saturn1

Help
Log off

Telephony Administration
Start ASA

Downiaad ASA

Data BackupRestore:
Backup Now

Schechle Backup

View Backup Log

iewRestore Data

Alarms and Nofification

~View Current Alarms.
Server

igw Summery St
igw Process St
irterchange Server!

Busy Out Server
Release Server
Shutdown This Server
Set Server Time/Timezane.
Security

EnableDisable Modem
Startsstop FTP Server
View License Status
intall License
install Avaya Authertication
Set LAN Security
—nstal Avaya Root Certfcate
sump
Contigure Trap Destinations:
Startistop Master Agert
Diagnostics

Vigw Restert Causes
View System Logs
View Temperaturefvotage

View Summary Status

SERVER STATUS

Duplicated? yes
Standby Busied? no
Standby Refreshed? yes
Standby Shadowing: on
Duplication Link: up
Elapsed Time since Init/Interchange: 0d 21:24:00

saturn? saturni
Mode: Active Mode: Standby
Major hlarns: yes Major Alarms: no
Minor hlarms: yes Minor Alarns: yes
Control Network: 4 / 4 Control Network: 4 / 4
Server Hardvare: okay Server Hardvare: okay
Processes: okay Processes: okay

© Display repeatedly every [05 7] seconds

View About This Screen





19
Perform this step only if this is an UPGRADE to a S8700 External Communications Controller (ECC) configuration (if you have G700’s with Local Spare Processors (LSP) as part of this configuration):

If this is an ECC configuration, upgrade all of the media gateways on the system at this time.  If they have already been upgraded, then you can start the LSP (from a telnet session to the LSP) by executing “start –ac “ from the bash (Linux shell) on the media gateway (LSP).  

Once the busy-out feature is implemented for LSP Media Gateways you will just need to release these systems back into service

NOTE: If you need to find the IP address of the media gateway in order to telnet, just execute “list media-gateway” from your SAT/ASA session.

NOTE: Patching steps 5-13 can be used to upgrade the Local Spare Processor (LSP) modules.  Ignore any instructions to busy-out or interchange, as these are not applicable to the LSPs.  
[image: image27.png]T Ciese New Tenplte

Fi Use Tenplte

== | Generate Call Accour
Report
ExportData
Iport Data
4 Find and Replace

G Stat Ematon

wechon

s





[image: image28.png]




Patching Instructions for New Installations of Avaya S8700 Media Servers

A New Installation means “brand new” (not previously installed) S8700 sites and existing G3r sites that are undergoing a migration to the S8700 platform.

Step
Instructions
Example Screens

1
Make sure you have run “Configure Server” under the Server Configuration and Upgrades heading from the media server web pages, which can be accessed from a web browser (see step 2).  In addition, the latest MultiVantage software load and a valid authentication file must be installed prior to executing these steps. 

REMEMBER:  DO NOT WIRE THE SERVERS UNTIL COMPLETING THESE STEPS!

NOTE:  To verify which software version you are running, select “View Software Version” from the media server web pages.  This can be found under the Server Configuration and Upgrades heading.




2
Start a web browser and specify the media server modem PPP IP address as the URL if logging in over a modem interface. Login using the appropriate login.

OR 

If there is LAN connectivity to the media server, start a web browser and specify the DNS name or LAN IP address of the primary server as the URL.  Login using the appropriate login. 

OR

If you are on-site and do not have LAN connectivity to the media server, connect a laptop to the primary server with a crossover cable, start a web browser on the laptop and specify 192.11.13.6 as the URL and login using the appropriate login.


[image: image29.png]WARNING: This s
access is a violati
reasons. By proce

This site ermploys ¢
certificate” provide
certificate, you will
offered this certifics
Screen" for instruct

&

AVAYA

communication

Information you exchange with i site cannol be viewed o1
changed by athers. However,there is » problem with the sie's
secuiy cetiicate.

The secuty cartcate was issued by a company you have.
ot chosen ta st View the cerlficale to determine whether
youwant to st the cartiying autharty.

@ The secuity certcate date s valid

The name on the secuiy certicate is invald o doss ot
malch the name of the ste

Do you want o proceed?

e ViewCoticae

oses. Unauthorized
strafive and security

have a "security
loes not have this

s certficate. If you are
b. Click "About This





[image: image30.png]AVAYA

communication

WARNING: This system is restricted to authorized users for business purposes. Unauthorized
access is a violation of the law. This system may be monitored for administrative and security
reasons. By proceeding, you consen {o this monitoring.

Username: init

Password: f==<f






3
An Avaya Authentication file needs to be installed on the server prior to applying a patch.   Use “Upload Files to Server (via browser)” under the Miscellaneous heading from the media server web pages, or manually FTP (using binary transfer mode) the authentication file onto the media server.
In order to install the authentication file perform the following set of instructions:

Select “Install Avaya Authentication” under the Security heading from the media server web pages, and then choose “Install”.  You should get confirmation that the file was installed successfully.  Ignore any indication that it could not verify the license.  This will be true until after the installation is complete and your new license is installed.
[image: image31.png]| AlaTNS and ROTMcation:

View Curtet Alme
Server i i

T sy St Install Avaya Authentication
View Process St

terchange Servers

Busy Ot Server

Release Server Install new Avaya authentication file on the server.

Shtcon T Server

St Server TimefTimezone

Security Install About This Screen

Startsstop FTP Server
View License Status

Set LAN Security
—nstal Avaya Root Certfite
sump
Contigure Trap Destinations:
Startistop Master Agert
Diagnostics

Vigw Restert Causes
View System Logs
View Temperatureivottage

Miscellaneous
EnableDisable Ethernat Swich Parts
Uploed Fils to Server (ia browser)
Downiaed Files ta Server (from wek)
Server Configuration and Upgrades.

iew Software Version
Contigure Server
install New Software Release.
Make Server Upgrace Permanert
igw Pertton Status

1PS Firmware Upgrades
View Serial Number(s)





[image: image32.png]>lart
Downiaad ASA
Data BackupRestore:
Backup Now
Schechle Backup
View Backup Log
~ViewRestore Data
Alarms and Nofification
~View Current Alarms.
Server
Vigw Summery Status
igw Process Stalus.
iterchange Servers
Busy Out Server
Release Server
Shutdown This Server
Set Server Time/Timezane.
Security
EnableDisable Modem
Startsstop FTP Server
View License Status
intall License
install Avaya Authertication
Set LAN Security
—nstal Avaya Root Certfcate
sump
Contigure Trap Destinations:
Startistop Master Agert
Diagnostics
Vigw Restert Causes
View System Logs
View Temperatureivottage

Miscellaneous

EnableDisable Ethernat Swich Parts

Uploed Fils to Server (ia browser)

Downiaed Files ta Server (from wek)

Server Configuration and Upgrades.
iew Software Version

Confioure Server

Install Avaya Authentication

Authentication installation successful

Passuord install file used: /var/home/fcp/Pilsneri.pud
Password file installation successfull

About This Screen





4
You will need to apply the patch before activating the system. 

From the laptop, FTP the patch to the primary media server via manual FTP or using the media server web pages “Upload Files to Server (via browser)” command under the Miscellaneous heading.  If you are downloading from an external Avaya web site you can use the media server web pages “Download Files to Server (from web)” command under the Miscellaneous heading to complete this step. 

MAKE SURE TO USE BINARY TRANSFER IF MANUALLY FTP’ING THE FILE!
[image: image33.png]telease Server

fuddown This Server i
et Server Time/Timezone SeloadiRlIeitolsenien

Socurity
rableDisatle Modem This page allows data files to be loaded onto the Server from the machine that your browser is
StertiStop FTP Server running on. In one or more of the fields below, enter the file name(s) to be sent to the server

[CAOT.0.080.4-4227 tar.g _ Brawse.

Browse.
File(s) to upload:

StartiStop Master Agert Browse.

Diagnostics Browse.

Vigw Restert Causes
LOAD FILE About This Screen

Miscellaneous
nableiDisable Ethernet Swich Parts
Jload Files to Server (via browser)
owrioed Fes to Server (romi)
Server Configuration and Upgr

lew Software Versian
ontigure Server
st New Software Release

1PS Firmware Upgrades
few Seril Number()

few PS Version

owrioed PSIFirmware

few Dowrioad Status

clivate PSI Lngrace

lew Firmware Activation Stetus





[image: image34.png]Release Server
Shutdown This Server
Set Server Time/Timezane.
Security

-Enable/Disable Modem The following files have been successfully uploaded to the server.
Startsstop FTP Server

Upload Files to Server

File Size
01.0.0604-4227 tar gz | 2873403

About This Screen

Miscellaneous
- EnableiDisable Ethernet Switch Parts

Uploed Fils to Server (ia browser)

Downiaed Files ta Server (from wek)

Server Configuration and Upgrades.
“View Software Version

Contigure Server

install New Software Release.

Make Server Upgrace Permanert

igw Pertton Status






5
Start a telnet session to the primary media server.  

From the telnet session, change directories to /var/home/ftp by executing the following at the command prompt:

> cd /var/home/ftp

To verify the file was uploaded to the media server, execute the following from the telnet session:

> ls –ltr

This will list the files in this directory, the last file shown should be the file you just ftp’ed which will be in the format of <patch>.tar.gz
[image: image35.png][commandpromet e [= ]

iorosoft Windows 2000 [Uersion 5.00.21951

KC> Copyright 1985-2088 Microsoft Corp.
s\Otelnet 192.11.13.6





[image: image36.png][ Command Prompt - telnet saturn2 =10l x]|

raft@saturn2> cd /var/home/FED





[image: image37.png]/INNT\System32\telnet.exe =10l x]|

susers 2873403 May 1 15:19 B1.8.868.4-4227.tar.gz






6
In order to install the patch execute the following from the telnet session:

>  patch_show

This will show any previous patches installed on the system.  Since this is a new install, there should not be any patches installed.

NOTE: DO NOT REMOVE PREVIOUS PATCHES WHEN APPLYING THE MOST CURRENT PATCH !

Now to install:

>sudo patch_install <patch>.tar.gz

To verify the patch was installed:

>  patch_show


[image: image38.png][ Command Prompt - telnet saturn2

-1o) x|
raft@saturn2> patch_shou

Patches are shoun in the order applied

oad-Patch Id Status

raft@saturn> _





[image: image39.png][ Command Prompt - telnet saturn2 =10l x]|

raft@saturn2> sudo patch_install O1.8.060.4-4227.tar.gz





[image: image40.png][ Command Prompt - telnet saturn2

raft@saturn2> patch_show

=lolx|

Patches are shoun in the order applied
oad-Patch Id Status

1.9.960.4-4227 not applied one-way talk patch on ISDN trunk
Gith EC5@8 int

raft@saturn2>






7
In order to apply the patch, execute the following from the telnet session:

>  sudo patch_apply <patch>

NOTICE: you do not use the .tar.gz extension on the back of the file name

NOTE:  Ignore the indication that “/opt/ecs/sbin/drestart 1 4 command failed”

NOTE:  when you apply the patch, the standby media server only will go through a software reset system 4. 

You will experience a delay in response when typing on the command line until the restart/reset has completed.
[image: image41.png]=lo/x|

raft@saturn2> patch_show

Patches are shoun in the order applied

Status Patch-description

1.
Gith EC5@8 int

not applied one-way talk patch on ISDN trunk

raft@saturn2> sudo patch_apply B1.8.868.4-4227
Successfully applied the patch
Executing post-patch commands
-3 Jopt/ecs/shinsdrestart 1 4 command passed







8
Once system response has returned, from the telnet session, verify the patch was applied.    Execute the following:

>  patch_show

This should show the status of the patch as “applied”.
[image: image42.png]=lo/x|

Patches are shoun in the order applied
oad-Patch Id

1.9.960.4-4227
Gith EC5@8 int

one-way talk patch on ISDN trunk

raft@saturn> _






9
From the web browser, choose “View Software Version” from the Server Configuration and Upgrades heading, and verify the patch is shown applied to the system.  

This information is shown directly under the Release String in the middle of the page. 


[image: image43.png]Release Server
Shutdown This Server
Set Server Time/Timezane.
Security

EnableDisable Modem
Startsstop FTP Server

Miscellaneous
- EnableiDisable Ethernet Switch Parts

Uploed Fils to Server (ia browser)

Downiaed Files ta Server (from wek)

Server Configuration and Upgrades.
“View Software Version

Contigure Server

install New Software Release.

Make Server Upgrace Permanert

igw Pertton Status

View Software Version

Operating systen
Built

Contains
Reports as

Release String
01.0.060.4-4227

Translation Saved

License Installed

About This Screen

Linuz 2.2.17-14.1s15 1686 unknovn

Mar 8 16721 2002
01.0.060.4
RO11:.01.0.060.4
S8700-011-01.0.060.4
applied

May 113:13
Apr 17 16:19

one-way talk patch on ISDN trunks with E






10
If connected via laptop interface:

Now disconnect the laptop crossover cable from the current server.  You are now not connected to any media server.  From the MS-DOS command window execute:

>   arp –d 192.11.13.6

This command flushes the route table in the laptop before connecting to the secondary media server.

Connect the crossover cable to the other media server (the secondary) and telnet to this server by executing the following from the MS-DOS command window:

> telnet 192.11.13.6

NOTE:  Be patient if this connection is slow; give it a few minutes.

OR 

If connecting via customer LAN:  

Telnet or connect to the other media server (the secondary) over the customer’s LAN IP address 

OR

If connecting via the modem:

From the current session execute the following from the server prompt:

> telnet <server_name>-dup


[image: image44.png]KC> Copyright 1985-2088 Microsoft Corp.

-1o) x|
\varp —d 192.11.13.6





[image: image45.png][commandpromet e [= ]

iorosoft Windows 2000 [Uersion 5.00.21951

KC> Copyright 1985-2088 Microsoft Corp.
s\Otelnet 192.11.13.6






11
Repeat steps 3-8 for this server.  It will then be safe to wire your system at this point.  One of the systems will back off from active after you connect the fiber and Ethernet cables. You are now complete with the MultiVantage software patching.


[image: image46.png]This Server: [2] saturn2
Duplicate Server: [1] saturn1

Help
Log off

Telephony Administration
Start ASA

Downiaad ASA

Data BackupRestore:
Backup Now

Schechle Backup

View Backup Log

iewRestore Data

Alarms and Nofification

~View Current Alarms.
Server

igw Summery St
igw Process St
irterchange Server!

Busy Out Server
Release Server
Shutdown This Server
Set Server Time/Timezane.
Security

EnableDisable Modem
Startsstop FTP Server
View License Status
intall License
install Avaya Authertication
Set LAN Security
—nstal Avaya Root Certfcate
sump
Contigure Trap Destinations:
Startistop Master Agert
Diagnostics

Vigw Restert Causes
View System Logs
View Temperaturefvotage

View Summary Status

SERVER STATUS

Duplicated? yes
Standby Busied? no
Standby Refreshed? yes
Standby Shadowing: on
Duplication Link: up
Elapsed Time since Init/Interchange: 0d 21:24:00

saturn? saturni
Mode: Active Mode: Standby
Major hlarns: yes Major Alarms: no
Minor hlarms: yes Minor Alarns: yes
Control Network: 4 / 4 Control Network: 4 / 4
Server Hardvare: okay Server Hardvare: okay
Processes: okay Processes: okay

© Display repeatedly every [05 7] seconds

View About This Screen





12
If you have G700’s with Local Spare Processors (LSP) as part of this configuration:

If this is an External Communications Controller (ECC) configuration, upgrade all of the media gateways (LSPs) on the system at this time.  If they have already been upgraded, then you can start the LSP (from a telnet session to the LSP) by executing “start –ac “ from the bash (Linux shell) on the media gateway (LSP).  

Once the busy-out feature is implemented for LSP Media Gateways you will just need to release these systems back into service

NOTE: If you need to find the IP address of the media gateway in order to telnet, just execute “list media-gateway” from a SAT/ASA session.

NOTE: Patching steps 3-8 can be used to upgrade the LSP modules. 
[image: image47.png]T Ciese New Tenplte

Fi Use Tenplte

== | Generate Call Accour
Report
ExportData
Iport Data
4 Find and Replace

G Stat Ematon

wechon

s





[image: image48.png]




Patching Instructions for Upgrades to Avaya S8300 Media Servers

An Upgrade means existing (previously installed) S8300 sites ONLY

These instructions apply to S8300 Internal Communications Controller (ICC) configurations only.

For MultiVantageTM 1.1 Local Spare Processor (LSP) configurations are not supported with S8300 ICC configurations. 

Step
Instructions
Example Screens

1
Perform a Backup of all data sets using “Backup Now” under the Data Backup/Restore heading on the media server web pages, which can be accessed from a web browser. 

You can backup to an FTP server that is on the customer network or to the media server hard drive.

NOTE: you cannot backup translations via e-mail due to customer restrictions on their e-mail servers. 
[image: image49.png]AVAYA

This Server: [1] saturn
Duplicate Server: [2] saturn2

Help
Log off

Telephony Administration
ort ASA
~Dowrload ASA
Data BackupRestore:
—Backup Now
~Schedle Backup
—View Backup Log
~ViewRestore Data
Alarms and Nofification
~View Current Alarms.
Server
“View Summary Status
~View Pracess Status
frchange Servers
—Busy Out Server
~Release Server
—-Shutdown This Server
—-Set Server TinefTimezone
Security
~EnableiDisatle Madem
ertStop FTP Server
—View Lizense Status
—nstal Licznse
—instal Avaya Authertication
—-Set LAN Securty
~Enatle Tripwire
ipwire Commands
—nstal vaya Root Certfcate
sump
Contigure Trap Destinations:
ertStop Master Agent
iagnostics
View Restart Causes

Backup Now

Data Sets:
¥ Avaya Call Processing (ACP) Translations
& Save ACP translations prior to backup
© Do NOT save ACP translations prior to backup
¥ Server and System Files
¥ Security Files

Backup Method:

IFTP User Name: [swadrin Password: ==
Host Name: [dura Directory: [e/swadmin/satum/trans
© Email  User Name: at Domain Name:
Mail Server:

¢ LocalPC Card  Retain data sets at destination

Encryption:
I Encrypt backup using pass phrase

Start Backup About This Screen






2
From the laptop, FTP the patch to the primary media server via manual FTP or using the media server web pages “Upload Files to Server (via browser)” command under the Miscellaneous heading.  If you are downloading from an external Avaya web site you can use the media server web pages “Download Files to Server (from web)” command under the Miscellaneous heading to complete this step. 

MAKE SURE TO USE BINARY TRANSFER IF MANUALLY FTP’ING THE FILE!
[image: image50.png]telease Server

fuddown This Server i
et Server Time/Timezone SeloadiRlIeitolsenien

Socurity
rableDisatle Modem This page allows data files to be loaded onto the Server from the machine that your browser is
StertiStop FTP Server running on. In one or more of the fields below, enter the file name(s) to be sent to the server

[CAOT.0.080.4-4227 tar.g _ Brawse.

Browse.
File(s) to upload:

StartiStop Master Agert Browse.

Diagnostics Browse.

Vigw Restert Causes
LOAD FILE About This Screen

Miscellaneous
nableiDisable Ethernet Swich Parts
Jload Files to Server (via browser)
owrioed Fes to Server (romi)
Server Configuration and Upgr

lew Software Versian
ontigure Server
st New Software Release

1PS Firmware Upgrades
few Seril Number()

few PS Version

owrioed PSIFirmware

few Dowrioad Status

clivate PSI Lngrace

lew Firmware Activation Stetus





[image: image51.png]Release Server
Shutdown This Server
Set Server Time/Timezane.
Security

-Enable/Disable Modem The following files have been successfully uploaded to the server.
Startsstop FTP Server

Upload Files to Server

File Size
01.0.0604-4227 tar gz | 2873403

About This Screen

Miscellaneous
- EnableiDisable Ethernet Switch Parts

Uploed Fils to Server (ia browser)

Downiaed Files ta Server (from wek)

Server Configuration and Upgrades.
“View Software Version

Contigure Server

install New Software Release.

Make Server Upgrace Permanert

igw Pertton Status






3
Start a telnet session to the primary media server.  

From the telnet session, change directories to /var/home/ftp by executing the following at the command prompt:

> cd /var/home/ftp

To verify the file was uploaded to the media server, execute the following from the telnet session:

> ls –ltr

This will list the files in this directory, the last file shown should be the file you just ftp’ed which will be in the format of <patch>.tar.gz
[image: image52.png][commandpromet e [= ]

iorosoft Windows 2000 [Uersion 5.00.21951

KC> Copyright 1985-2088 Microsoft Corp.
s\Otelnet 192.11.13.6




[image: image53.png][ Command Prompt - telnet saturn2 =10l x]|

raft@saturn2> cd /var/home/FED





[image: image54.png]/INNT\System32\telnet.exe =10l x]|

susers 2873403 May 1 15:19 B1.8.868.4-4227.tar.gz






4
In order to install the patch execute the following from the telnet session:

>  patch_show

This will show any previous patches installed on the system.  

NOTE: DO NOT REMOVE PREVIOUS PATCHES WHEN APPLYING THE MOST CURRENT PATCH !

Now to install:

>sudo patch_install <patch>.tar.gz

To verify the patch was installed:

>  patch_show


[image: image55.png][ Command Prompt - telnet saturn2

-1o) x|
raft@saturn2> patch_shou

Patches are shoun in the order applied

oad-Patch Id Status

raft@saturn> _





[image: image56.png][ Command Prompt - telnet saturn2 =10l x]|

raft@saturn2> sudo patch_install O1.8.060.4-4227.tar.gz





[image: image57.png][ Command Prompt - telnet saturn2

raft@saturn2> patch_show

=lolx|

Patches are shoun in the order applied
oad-Patch Id Status

1.9.960.4-4227 not applied one-way talk patch on ISDN trunk
Gith EC5@8 int

raft@saturn2>






5
In order to apply the patch, execute the following from the telnet session:

>  sudo patch_apply <patch>

NOTICE: you do not use the .tar.gz extension on the back of the file name

NOTE:  Ignore the indication that “/opt/ecs/sbin/drestart 1 4 command failed”

NOTE:  when you apply the patch, the media server will go through a software reset system 4. 

You will experience a delay in response when typing on the command line until the restart/reset has completed.
[image: image58.png]=lo/x|

raft@saturn2> patch_show

Patches are shoun in the order applied

Status Patch-description

1.
Gith EC5@8 int

not applied one-way talk patch on ISDN trunk

raft@saturn2> sudo patch_apply B1.8.868.4-4227
Successfully applied the patch
Executing post-patch commands
-3 Jopt/ecs/shinsdrestart 1 4 command passed







6
Once system response has returned, from the telnet session, verify the patch was applied.    Execute the following:

>  patch_show

This should show the status of the patch as “applied”.
[image: image59.png]=lo/x|

Patches are shoun in the order applied
oad-Patch Id

1.9.960.4-4227
Gith EC5@8 int

one-way talk patch on ISDN trunk

raft@saturn> _






Patching Instructions for New Installations of Avaya S8300 Media Servers

A New Installation means “brand new” (not previously installed) S8300 sites.

These instructions apply to S8300 Internal Communications Controller (ICC) configurations only.

For MultiVantageTM 1.1 Local Spare Processor (LSP) configurations are not supported with S8300 ICC configurations. 

Step
Instructions
Example Screens

1
Make sure you have run “Configure Server” under the Server Configuration and Upgrades heading from the media server web pages, which can be accessed from a web browser (see step 2).  In addition, the latest MultiVantage software load and a valid authentication file must be installed prior to executing these steps. 

REMEMBER:  DO NOT WIRE THE SERVERS UNTIL COMPLETING THESE STEPS!

NOTE:  To verify which software version you are running, select “View Software Version” from the media server web pages.  This can be found under the Server Configuration and Upgrades heading.




2
Start a web browser and specify the media server modem PPP IP address as the URL if logging in over a modem interface. Login using the appropriate login.

OR 

If there is LAN connectivity to the media server, start a web browser and specify the DNS name or LAN IP address of the primary server as the URL.  Login using the appropriate login. 

OR

If you are on-site and do not have LAN connectivity to the media server, connect a laptop to the primary server with a crossover cable, start a web browser on the laptop and specify 192.11.13.6 as the URL and login using the appropriate login.


[image: image60.png]WARNING: This s
access is a violati
reasons. By proce

This site ermploys ¢
certificate” provide
certificate, you will
offered this certifics
Screen" for instruct

&

AVAYA

communication

Information you exchange with i site cannol be viewed o1
changed by athers. However,there is » problem with the sie's
secuiy cetiicate.

The secuty cartcate was issued by a company you have.
ot chosen ta st View the cerlficale to determine whether
youwant to st the cartiying autharty.

@ The secuity certcate date s valid

The name on the secuiy certicate is invald o doss ot
malch the name of the ste

Do you want o proceed?

e ViewCoticae

oses. Unauthorized
strafive and security

have a "security
loes not have this

s certficate. If you are
b. Click "About This





[image: image61.png]AVAYA

communication

WARNING: This system is restricted to authorized users for business purposes. Unauthorized
access is a violation of the law. This system may be monitored for administrative and security
reasons. By proceeding, you consen {o this monitoring.

Username: init

Password: f==<f






3
An Avaya Authentication file needs to be installed on the server prior to applying a patch.   Use “Upload Files to Server (via browser)” under the Miscellaneous heading from the media server web pages, or manually FTP (using binary transfer mode) the authentication file onto the media server.
In order to install the authentication file perform the following set of instructions:

Select “Install Avaya Authentication” under the Security heading from the media server web pages, and then choose “Install”.  You should get confirmation that the file was installed successfully.  Ignore any indication that it could not verify the license.  This will be true until after the installation is complete and your new license is installed.
[image: image62.png]| AlaTNS and ROTMcation:

View Curtet Alme
Server i i

T sy St Install Avaya Authentication
View Process St

terchange Servers

Busy Ot Server

Release Server Install new Avaya authentication file on the server.

Shtcon T Server

St Server TimefTimezone

Security Install About This Screen

Startsstop FTP Server
View License Status

Set LAN Security
—nstal Avaya Root Certfite
sump
Contigure Trap Destinations:
Startistop Master Agert
Diagnostics

Vigw Restert Causes
View System Logs
View Temperatureivottage

Miscellaneous
EnableDisable Ethernat Swich Parts
Uploed Fils to Server (ia browser)
Downiaed Files ta Server (from wek)
Server Configuration and Upgrades.

iew Software Version
Contigure Server
install New Software Release.
Make Server Upgrace Permanert
igw Pertton Status

1PS Firmware Upgrades
View Serial Number(s)





[image: image63.png]>lart
Downiaad ASA
Data BackupRestore:
Backup Now
Schechle Backup
View Backup Log
~ViewRestore Data
Alarms and Nofification
~View Current Alarms.
Server
Vigw Summery Status
igw Process Stalus.
iterchange Servers
Busy Out Server
Release Server
Shutdown This Server
Set Server Time/Timezane.
Security
EnableDisable Modem
Startsstop FTP Server
View License Status
intall License
install Avaya Authertication
Set LAN Security
—nstal Avaya Root Certfcate
sump
Contigure Trap Destinations:
Startistop Master Agert
Diagnostics
Vigw Restert Causes
View System Logs
View Temperatureivottage

Miscellaneous

EnableDisable Ethernat Swich Parts

Uploed Fils to Server (ia browser)

Downiaed Files ta Server (from wek)

Server Configuration and Upgrades.
iew Software Version

Confioure Server

Install Avaya Authentication

Authentication installation successful

Passuord install file used: /var/home/fcp/Pilsneri.pud
Password file installation successfull

About This Screen





4
You will need to apply the patch before activating the system. 

From the laptop, FTP the patch to the primary media server via manual FTP or using the media server web pages “Upload Files to Server (via browser)” command under the Miscellaneous heading.  If you are downloading from an external Avaya web site you can use the media server web pages “Download Files to Server (from web)” command under the Miscellaneous heading to complete this step. 

MAKE SURE TO USE BINARY TRANSFER IF MANUALLY FTP’ING THE FILE!
[image: image64.png]telease Server

fuddown This Server i
et Server Time/Timezone SeloadiRlIeitolsenien

Socurity
rableDisatle Modem This page allows data files to be loaded onto the Server from the machine that your browser is
StertiStop FTP Server running on. In one or more of the fields below, enter the file name(s) to be sent to the server

[CAOT.0.080.4-4227 tar.g _ Brawse.

Browse.
File(s) to upload:

StartiStop Master Agert Browse.

Diagnostics Browse.

Vigw Restert Causes
LOAD FILE About This Screen

Miscellaneous
nableiDisable Ethernet Swich Parts
Jload Files to Server (via browser)
owrioed Fes to Server (romi)
Server Configuration and Upgr

lew Software Versian
ontigure Server
st New Software Release

1PS Firmware Upgrades
few Seril Number()

few PS Version

owrioed PSIFirmware

few Dowrioad Status

clivate PSI Lngrace

lew Firmware Activation Stetus





[image: image65.png]Release Server
Shutdown This Server
Set Server Time/Timezane.
Security

-Enable/Disable Modem The following files have been successfully uploaded to the server.
Startsstop FTP Server

Upload Files to Server

File Size
01.0.0604-4227 tar gz | 2873403

About This Screen

Miscellaneous
- EnableiDisable Ethernet Switch Parts

Uploed Fils to Server (ia browser)

Downiaed Files ta Server (from wek)

Server Configuration and Upgrades.
“View Software Version

Contigure Server

install New Software Release.

Make Server Upgrace Permanert

igw Pertton Status






5
Start a telnet session to the primary media server.

From the telnet session, change directories to /var/home/ftp by executing the following at the command prompt:

> cd /var/home/ftp

To verify the file was uploaded to the media server, execute the following from the telnet session:

> ls –ltr

This will list the files in this directory, the last file shown should be the file you just ftp’ed which will be in the format of <patch>.tar.gz
[image: image66.png][commandpromet e [= ]

iorosoft Windows 2000 [Uersion 5.00.21951

KC> Copyright 1985-2088 Microsoft Corp.
s\Otelnet 192.11.13.6




[image: image67.png][ Command Prompt - telnet saturn2 =10l x]|

raft@saturn2> cd /var/home/FED





[image: image68.png]/INNT\System32\telnet.exe =10l x]|

susers 2873403 May 1 15:19 B1.8.868.4-4227.tar.gz






6
In order to install the patch execute the following from the telnet session:

>  patch_show

This will show any previous patches installed on the system.  Since this is a new install, there should not be any patches installed.

NOTE: DO NOT REMOVE PREVIOUS PATCHES WHEN APPLYING THE MOST CURRENT PATCH !

Now to install:

>sudo patch_install <patch>.tar.gz

To verify the patch was installed:

>  patch_show


[image: image69.png][ Command Prompt - telnet saturn2

-1o) x|
raft@saturn2> patch_shou

Patches are shoun in the order applied

oad-Patch Id Status

raft@saturn> _





[image: image70.png][ Command Prompt - telnet saturn2 =10l x]|

raft@saturn2> sudo patch_install O1.8.060.4-4227.tar.gz





[image: image71.png][ Command Prompt - telnet saturn2

raft@saturn2> patch_show

=lolx|

Patches are shoun in the order applied
oad-Patch Id Status

1.9.960.4-4227 not applied one-way talk patch on ISDN trunk
Gith EC5@8 int

raft@saturn2>







7
In order to apply the patch, execute the following from the telnet session:

>  sudo patch_apply <patch>

NOTICE: you do not use the .tar.gz extension on the back of the file name

NOTE:  Ignore the indication that “/opt/ecs/sbin/drestart 1 4 command failed”

NOTE:  when you apply the patch, the media server will go through a software reset system 4. 

You will experience a delay in response when typing on the command line until the restart/reset has completed.
[image: image72.png]=lo/x|

raft@saturn2> patch_show

Patches are shoun in the order applied

Status Patch-description

1.
Gith EC5@8 int

not applied one-way talk patch on ISDN trunk

raft@saturn2> sudo patch_apply B1.8.868.4-4227
Successfully applied the patch
Executing post-patch commands
-3 Jopt/ecs/shinsdrestart 1 4 command passed







8
Once system response has returned, from the telnet session, verify the patch was applied.    Execute the following:

>  patch_show

This should show the status of the patch as “applied”.
[image: image73.png]=lo/x|

Patches are shoun in the order applied
oad-Patch Id

1.9.960.4-4227
Gith EC5@8 int

one-way talk patch on ISDN trunk

raft@saturn> _






© 2002 Avaya Inc. All Rights Reserved.  Avaya and the Avaya Logo are trademarks of Avaya Inc. or Avaya ECS Ltd., a wholly owned subsidiary of Avaya Inc. and may be registered in the US and other jurisdictions.  All trademarks identified by ® and ™ are registered trademarks or trademarks, respectively, of Avaya Inc.  All other registered trademarks or trademarks are property 

of their respective owners.
28

[image: image74.png][image: image75.png][image: image76.png]AVAYA



